Please read the following information to answer any injector or installation questions.

1.  The new fuel injector nozzle is shorter than the stock injector. It is like this on purpose to increase flow and fuel atomization. The new injector will look shorter however the body of the injector is the same size and installs exactly the same as the factory injector.

2.  If you installed a larger than factory injector upon initial startup the engine will run rich. The new injector flows more fuel (if it isn’t a stock size replacement injector). The amount of fuel supplied to the engine needs to be reduced through the use of a fuel controller (i.e. Power Commander).

3.  Dyno tuning is highly recommended to optimize engine performance after installing your new injector.

Fuel Injector Installation Instructions

- Remove front plastics, fuel tank, and plastic guards covering engine and battery.

- Unclip the wire harness from the injector, and remove the fuel line from the fuel rail.

- Remove the two screws that hold the fuel rail in place. (These are usually very tight from the factory)

- Remove the fuel rail from the throttle body. The fuel injector fits snug into the fuel rail, so it will take a little pressure to remove.

- For all machines except KFX 450 skip to step 6. If you purchased a KFX injector the stock lower rubber o-ring will be re-used on the lower portion of the injector.

- Apply lube to the injector’s lower rubber donut and fuel rail O-rings to help ease installation. Insert the injector back into the throttle body.

- Slide the fuel rail over the top of the injector and line up the bolt holes. Using the stock bolts tighten the fuel rail back into place.

- Connect the supplied wire adapter onto the injector which then plugs into the ATV’s wire harness (or into where an aftermarket fuel controller connects).

- You are now finished. Re-install the fuel tank and fenders.

For questions feel free to email: geisickperformance@gmail.com or Call 970-768-7168 for assistance.