Raptor/Rhino 700 Cylinder Head Stud Kit

Installation Instructions

- Thread the 4 long studs into the cylinder until bottomed out. These only need to be snugged into the cylinder and do not need to be torqued down. Also make sure to install studs with the Allen side facing up.

- Install the two short studs into the cylinder head. Thread in until bottomed and make sure Allen side is facing out.

- Install the head with a new head gasket.

- Install the new washers and apply assembly lube to threads of the studs. Then install the nuts

- Torque head studs in steps with a final torque of 40 ft/lbs